SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO4:

Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling  standards

(BL2,BL6)

Assessment Tool Weightage: 30%

Annexure A

Industry Problem-Solving Task

- A star topology is ideal for a 20-person startup because all devices  connect to a central switch, making the network easy to manage  and troubleshoot. At the core sits a Layer 2/Layer 3 managed  switch (such as a Cisco Catalyst or Netgear ProSafe) with at least  24 ports to accommodate all workstations, printers, and VoIP  phones with room to grow. Each device connects to the switch  using Cat6 UTP cables, which support Gigabit speeds up to 100  meters — more than sufficient for any typical office floor plan.  The switch uplinks to a router/firewall (such as a Cisco RV series  or pfSense appliance) that handles internet access, DHCP, and  network address translation. A Wi-Fi access point connected to the  switch extends wireless coverage for laptops and mobile devices.

Answer:

Problem:
A 20-person startup needs a reliable and scalable office network for workstations, printers, VoIP phones, laptops, and mobile devices. The network should be easy to manage, troubleshoot, and expand while minimizing the effect of individual device or cable failures.

Proposed Solution:
Implement a Star Topology using a central 24-port managed Layer 2/Layer 3 switch. Connect all workstations, printers, and VoIP phones to the switch using Cat6 UTP cables. The switch should be connected to a router/firewall for Internet access, DHCP, NAT, and network security. A Wi-Fi access point can also be connected to the switch to provide wireless connectivity.

Network Structure:

Internet

|

Router / Firewall

|

24-Port Managed

Switch

___________|________

|             |            |          |

PC1    PC2      PC3       PC20

|             |           |            |

Cat6      Cat6   Cat6       Cat6

|

Wi-Fi Access Point

|

Laptops / Mobiles

Problem-Solving Benefits:

| Problem | Solution |
| --- | --- |
| Difficult network management | Central managed switch |
| Cable/device failure | Only the affected device is disconnected |
| Limited network expansion | Extra switch ports are available |
| Slow network connectivity | Cat6 with Gigabit Ethernet |
| Internet access | Router/firewall |
| Wireless connectivity | Wi-Fi access point |
| Security requirements | Firewall and managed network controls |

Key Advantage:
The main advantage is fault isolation. If the cable or network connection of one workstation fails, only that workstation is affected; the other 19 employees can continue working normally.

Conclusion:
The proposed star topology provides a reliable, manageable, scalable, and fault-tolerant network for the startup. It is suitable for current requirements and can easily accommodate additional employees and devices in the future.

2.A bank demands near-zero downtime, making a full or partial  mesh topology the right choice. In a full mesh, every core network  device — routers, core switches, and servers — connects directly  to every other device, so no single link or node failure can bring  down the network. For a bank, a partial mesh is typically deployed  at the core layer (connecting all core switches and firewalls to each  other) while the distribution and access layers use redundant  uplinks to at least two core switches. Dual enterprise-grade routers  (such as Cisco ASR or Juniper MX series) provide BGP-based  redundant internet connectivity. All links between core devices  use fiber optic cables — either single-mode for long distances  between buildings or multi-mode OM4 within the data center — because fiber is immune to electromagnetic interference, supports  very high bandwidth, and is essential for maintaining low-latency  financial transactions. Redundant power supplies, UPS systems,  and RAID storage arrays further complement the mesh design. The  justification is straightforward: when a link fails in a mesh, traffic  automatically reroutes through alternative paths in milliseconds  using protocols like OSPF or BGP, satisfying regulatory  requirements for uptime and data integrity.

Answer:

Problem Statement

A bank requires a highly reliable network with near-zero downtime because even a short network failure can interrupt financial transactions, online banking, and critical banking services. The network must continue operating even if a link or network device fails.

Proposed Solution

A full or partial mesh topology is suitable for the bank's core network. In a full mesh, every core device is directly connected to every other core device. In practice, a partial mesh is more commonly used because it provides redundancy while reducing cost and complexity.

Network Design

- Core Layer: Core switches, routers, and firewalls are interconnected using multiple redundant links.

- Distribution Layer: Distribution switches have redundant uplinks to at least two core switches.

- Internet Connectivity: Two enterprise-grade routers provide redundant Internet connections.

- Fiber Optic Links: Fiber cables are used between core devices because they provide high bandwidth, low latency, and immunity to electromagnetic interference.

- Backup Infrastructure: UPS systems, redundant power supplies, and RAID storage provide additional protection against hardware and power failures.

Basic Network Structure

Internet

/            \

Router 1      Router 2

|         \              / |

|          \             /  |

Firewall 1 ----- Firewall 2

|        \                  /|

|           \               / |

Core Switch 1 --- Core Switch 2

/         \                         /   \

/            \                      /     \

Distribution 1       Distribution 2

/            \                        /   \

Access Switches      Access Switches

|                                 |

Users / Servers / Banking Devices

Fault Tolerance

The major advantage of mesh topology is fault tolerance. If one network link fails, traffic can automatically use another available path. Therefore, a single link or device failure does not necessarily stop banking services.

Routing protocols such as OSPF or BGP can be used to determine alternative paths and reroute traffic when a failure occurs.

Why Mesh Topology is Suitable for a Bank

| Requirement | Mesh Solution |
| --- | --- |
| Near-zero downtime | Multiple redundant paths |
| Link failure protection | Traffic reroutes through alternate paths |
| High-speed transactions | High-bandwidth fiber connections |
| Internet reliability | Dual routers/Internet connections |
| Network security | Redundant firewalls |
| Hardware failure protection | Redundant devices and power supplies |
| Data availability | RAID and redundant storage |
| Future expansion | Additional redundant links can be added |

Conclusion

A partial mesh topology with redundant core switches, routers, firewalls, and fiber-optic connections is an appropriate solution for a banking network. It provides high availability, fault tolerance, low latency, and reliable connectivity. If one link or device fails, alternative paths allow network traffic to continue, helping the bank maintain uninterrupted critical services and meet strict availability requirements.

3.For a small classroom lab with a tight budget, bus topology offers  the lowest hardware cost because all computers share a single  common communication line — the bus — eliminating the need  for a central switch or hub. A single coaxial cable (RG-58, 50- ohm) or, in a modern variation, a single Cat5e/Cat6 cable daisy  chained through an inexpensive unmanaged hub runs along the  length of the room. Each computer connects to the backbone using  a T-connector and a BNC connector in a classic coaxial setup, with  terminators (50-ohm) placed at both ends of the cable to absorb  signals and prevent reflection. For a more contemporary low-cost  implementation, a single inexpensive 8- or 16-port unmanaged  switch replaces the terminators and uses Cat5e UTP patch cables  to each workstation — achieving bus-like simplicity at minimal  cost. The primary limitation is that the network is susceptible to  collisions and a single cable break disrupts all connected  machines, but for a supervised classroom environment with light  usage (internet browsing, file sharing), this trade-off is entirely  acceptable. No dedicated server is required; a teacher's PC can  share resources using built-in Windows/Linux file sharing.

Answer:

Problem Statement

A small classroom computer lab has a limited budget and requires a simple network for activities such as Internet browsing, file sharing, and basic communication between computers. The network should require minimum hardware and installation cost.

Proposed Solution

A Bus Topology is suitable for a small classroom lab because all computers share a common communication backbone. It requires fewer networking devices compared with topologies that depend on a central switch.

In a traditional bus network, a single coaxial cable acts as the backbone. Each computer connects to this backbone using a T-connector and BNC connector. 50-ohm terminators are placed at both ends of the cable to prevent signal reflection.

For a modern low-cost implementation, an inexpensive 8-port or 16-port unmanaged switch with Cat5e/Cat6 cables can be used to provide similar simple connectivity.

Basic Network Structure

Computer 1 ─┐

Computer 2 ─┤

Computer 3 ─┤──── Common Network Backbone

Computer 4 ─┤

Computer 5 ─┘

Traditional Bus Components

- RG-58 coaxial cable

- T-connectors

- BNC connectors

- 50-ohm terminators

- Network interface cards

Modern Low-Cost Alternative

Unmanaged Switch

/       /      |     \      \

PC1 PC2  PC3  PC4  PC5

The modern alternative uses Cat5e/Cat6 UTP cables connected to an inexpensive unmanaged switch.

Advantages

- Low Cost: Requires fewer networking components.

- Simple Installation: Suitable for a small number of computers.

- Easy Resource Sharing: Computers can share files and printers.

- No Dedicated Server: A teacher's PC can provide basic file-sharing services.

- Suitable for Light Usage: Works for basic browsing and classroom activities.

Limitations

- A failure in the main backbone can affect all connected computers.

- Traditional bus networks can experience data collisions.

- Performance decreases as more devices use the shared medium.

- Troubleshooting a backbone failure can be difficult.

- It has limited scalability compared with star topology.

Why It is Suitable for the Classroom

The classroom has a small number of computers, limited budget, and relatively light network traffic. Therefore, the low cost of a bus-style network can outweigh its limitations. Since the network is supervised and mainly used for browsing and file sharing, the reduced reliability is an acceptable trade-off.

Conclusion

A bus topology can be considered a cost-effective solution for a small classroom lab with limited networking requirements. It minimizes hardware costs and provides basic connectivity and resource sharing. However, for a new modern installation, an inexpensive unmanaged switch with Cat5e/Cat6 cables is generally more practical and reliable than a traditional coaxial bus network.

4.A large university campus serves thousands of users across  multiple buildings — lecture halls, labs, libraries, dormitories, and  administration — making a single topology inadequate. A hybrid  topology combines the best elements of multiple designs: a fiber  based ring or mesh at the campus backbone level for redundancy,  a star topology within each building for manageability, and bus or  tree structures within individual labs for cost-efficiency. At the  campus core, high-speed fiber optic cables (single-mode, 10 Gbps  or 40 Gbps) interconnect core routers and multilayer switches  using a ring or partial mesh arrangement, ensuring that no single  link failure isolates an entire building. Each building has its own  distribution switch connected to the campus core via dual fiber  uplinks. Within each building, a traditional star topology branches  from the distribution switch to floor-level access switches, which  then connect individual devices via Cat6 cables. This hierarchical  three-layer model — core, distribution, and access — is the  industry standard for enterprise and campus networks because  adding a new building simply means installing a new distribution  switch and running fiber to the core, with no disruption to existing  infrastructure. Wireless access points throughout the campus  connect via PoE (Power over Ethernet) to access switches,  eliminating the need for separate power cabling. The hybrid design  thus delivers redundancy at the backbone, simplicity at the  building level, and scalability campus-wide.

Answer:

Problem Statement

A large university campus has thousands of users distributed across multiple buildings such as lecture halls, laboratories, libraries, hostels, and administrative offices. Since the requirements of each area are different, using a single network topology would not be efficient. The network must provide high availability, easy management, scalability, and reliable connectivity.

Proposed Solution

A Hybrid Topology is the most suitable solution because it combines different network topologies according to the requirements of each part of the campus.

The proposed design uses:

- Ring or partial mesh at the campus backbone.

- Star topology inside individual buildings.

- Bus or tree structures where appropriate inside small laboratories.

Network Architecture

The campus backbone uses high-speed fiber-optic cables to connect core routers and multilayer switches. A ring or partial mesh arrangement provides multiple paths between important network devices.

Each building has a distribution switch connected to the campus core using dual fiber-optic uplinks. Inside each building, the distribution switch connects to floor-level access switches using a star arrangement.

Basic Network Structure

CAMPUS CORE

┌──────────  ─┐

│ Core Routers /         │

│ Multilayer Switches│

└───────────  ┘

/            |          \

/             |            \

Fiber       Fiber      Fiber

/                  |                \

Building A     Building B    Building C

Distribution  Distribution  Distribution

Switch        Switch        Switch

/        |       \      /    |       \      /     |       \

Floor Floor Floor  Floor Floor  Floor Floor

SW    SW    SW     SW    SW     SW    SW

|            |         |         |         |           |         |

PCs      PCs   APs    PCs   PCs    PCs   PCs

Three-Layer Architecture

1. Core Layer

- Connects different university buildings.

- Uses high-speed fiber-optic links.

- Uses a ring or partial mesh arrangement.

- Provides redundancy and high-speed communication.

2. Distribution Layer

- Each building has distribution switches.

- Provides dual connections to the campus core.

- Controls traffic between the building and campus backbone.

3. Access Layer

- Floor-level switches connect individual computers, printers, and other devices.

- Cat6 cables are used for wired devices.

- Wireless access points can be connected using Power over Ethernet (PoE).

Advantages

| Requirement | Hybrid Topology Solution |
| --- | --- |
| Campus-wide connectivity | Fiber-optic backbone |
| Link failure protection | Ring/partial mesh |
| Easy building management | Star topology |
| Scalability | Modular building-level design |
| High-speed communication | 10/40 Gbps fiber links |
| Wireless connectivity | PoE access points |
| Easy expansion | Add new distribution switch and fiber uplinks |
| Cost efficiency | Different topologies used according to requirements |

Problem-Solving Benefits

Redundancy: If one backbone link fails, traffic can use another available path.

Manageability: Star topology inside buildings makes individual device connections easier to monitor and troubleshoot.

Scalability: New buildings can be added by installing a distribution switch and connecting it to the campus core without redesigning the existing network.

Cost Efficiency: Expensive high-speed infrastructure is concentrated at the backbone, while simpler and less expensive solutions are used inside smaller areas.

Conclusion

A Hybrid Topology is the most appropriate solution for a large university campus because no single topology can efficiently satisfy all campus requirements. Combining ring/partial mesh at the backbone, star topology within buildings, and suitable local structures in laboratories provides redundancy, manageability, cost efficiency, and campus-wide scalability. This design can also support future expansion as the university adds new buildings, users, and network services.

5.Designing a network for a company with multiple departments  involves creating a structured and scalable architecture using  switches and routers to ensure efficient communication and  security. Each department (such as HR, Finance, Sales, and IT) is  connected through dedicated Layer 2 switches that form the access  layer, allowing devices like computers and printers to  communicate within the same department. These switches are then connected to a central Layer 3 switch or router, which acts as the  core layer and enables communication between different  departments through inter-VLAN routing. Virtual LANs (VLANs)  are configured to logically separate each department’s network,  improving security and reducing broadcast traffic. A router is also  used to connect the internal network to external networks such as  the internet, often with firewall functionality for security.  Redundant links and backup paths can be included to ensure  reliability, while proper IP addressing and subnetting are applied  for efficient network management. This hierarchical design  improves performance, enhances security, and allows easy  expansion as the company grows.

Answer:

Problem Statement

A company with multiple departments such as HR, Finance, Sales, and IT needs a network that provides efficient communication while maintaining security and controlling access between departments. The network should also be scalable so that new users and departments can be added easily.

Proposed Solution

A hierarchical network architecture using Layer 2 switches, Layer 3 switches/routers, VLANs, and a firewall is proposed.

Each department is assigned a separate VLAN. Department devices are connected to dedicated Layer 2 access switches. These switches connect to a central Layer 3 switch, which performs inter-VLAN routing and allows controlled communication between departments.

Network Architecture

Internet

|

Router / Firewall

|

Layer 3 Core Switch

/       |       |       \

/        |       |        \

VLAN 10    VLAN 20  VLAN 30   VLAN 40

|          |        |         |

HR Switch  Finance   Sales     IT Switch

|       Switch    Switch       |

PCs/      PCs/      PCs/        PCs/

Printers  Printers  Printers    Printers

VLAN Configuration

| Department | VLAN | Example Network |
| --- | --- | --- |
| HR | VLAN 10 | 192.168.10.0/24 |
| Finance | VLAN 20 | 192.168.20.0/24 |
| Sales | VLAN 30 | 192.168.30.0/24 |
| IT | VLAN 40 | 192.168.40.0/24 |

Each department operates in its own logical network. This prevents unnecessary broadcast traffic between departments and improves network organization.

Working Mechanism

1. Access Layer

Layer 2 switches connect computers, printers, and other end devices within each department.

2. VLANs

A separate VLAN is created for each department. For example, HR devices are placed in VLAN 10, while Finance devices are placed in VLAN 20.

3. Core Layer

A Layer 3 switch performs inter-VLAN routing, allowing devices from different departments to communicate when permitted.

4. Router and Firewall

The router connects the company's internal network to the Internet. Firewall rules can be used to prevent unauthorized access and protect sensitive departmental data.

5. Redundancy

Backup links and redundant network devices can be implemented to reduce downtime if a primary connection fails.

Advantages

- Improved Security: VLANs isolate departmental traffic.

- Reduced Broadcast Traffic: Broadcasts are limited to individual VLANs.

- Efficient Communication: Inter-VLAN routing enables controlled communication.

- Scalability: New users and departments can easily be added.

- Centralized Management: Network devices can be monitored and managed efficiently.

- Reliability: Redundant links provide backup connectivity.

- Better Performance: Traffic is organized according to departments.

Conclusion

A hierarchical network using Layer 2 access switches, VLANs, a Layer 3 core switch, and a router/firewall provides an efficient and secure solution for a multi-department company. VLAN segmentation improves security and reduces unnecessary traffic, while inter-VLAN routing enables controlled communication between departments. The design is also scalable, reliable, and easy to manage, making it suitable for a growing organization.

RUBRICS FOR CALCULATION

| Criteria |  | Weightage Exemplary (4) Proficient (3) Developing (2) Beginning (1) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Problem Analysis | 15% | Clearly identifies and deeply analyzes the industry problem with all factors | Good understanding with minor gaps | Basic understanding; some key aspects missing | Poor or incorrect understanding of the problem |
| Solution Design 20% |  | Innovative, practical, and industry relevant solution | Effective solution with minor limitations | Partially suitable solution | Inappropriate or unclear solution |
| Technical Implementation | 20% | Fully functional, accurate, and well-executed implementation | Mostly functional with minor errors | Partially implemented solution | Incomplete or non-functional implementation |
| Use of Tools & Technologies | 10% | Appropriate and advanced use of relevant tools and technologies | Correct use of tools | Limited or basic use of tools | Incorrect or no use of tools |
| Problem Solving Approach | 10% | Logical, systematic, and well-structured approach | Mostly logical approach | Somewhat disorganized approach | No clear approach |
| Innovation & Creativity | 10% | Highly creative and original solution | Some creativity | Limited originality | No creativity |
| Testing & Validation | 5% | Thorough testing with real-world scenarios | Adequate testing | Minimal testing | No testing |
| Documentation & Presentation | 5% | Clear, professional, and well documented | Good documentation | Basic documentation | Poor or no documentation |
| Teamwork / Collaboration | 5% | Excellent coordination and contribution (if team-based) | Good collaboration | Limited participation | No collaboration or contribution |