SIMATS ENGINEERING

ASSESSMENT TOOL 3

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO4:

Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling  standards

(BL2,BL6)

Assessment Tool Weightage: 30%

QUESTIONS: 5 Total Marks:25

Annexure A

Comparative Analysis

1. Star topology is generally more expensive than bus topology because it requires a central  device such as a switch or hub and more cabling to connect each node individually. In  contrast, bus topology uses a single backbone cable, making it low-cost and easier to install.  However, in terms of reliability, star topology is superior because failure of one node does  not affect others, whereas a failure in the main bus cable can bring down the entire network.  Performance is also better in star topology since data transmission is managed through a  central device, reducing collisions, while bus topology suffers from performance  degradation as more devices are added due to increased data traffic on the shared medium.

| Feature | Star Topology | Bus Topology |
| --- | --- | --- |
| Cost | More expensive because it requires a central switch/hub and separate cables for each device. | Less expensive because it uses a single backbone cable. |
| Installation | Requires more cabling and configuration. | Easy and simple to install. |
| Reliability | Highly reliable. Failure of one device does not affect the other devices. | Less reliable. Failure of the main backbone cable can affect the entire network. |
| Performance | Better performance because the central switch manages data transmission and reduces collisions. | Performance decreases as more devices are connected because all devices share the same cable. |
| Scalability | Easy to add or remove devices without affecting the rest of the network. | Adding more devices can increase traffic and reduce performance. |
| Maintenance | Easier to identify and troubleshoot individual device failures. | Troubleshooting can be difficult because all devices share the same backbone. |

2.Mesh topology offers very high fault tolerance because every node is connected to multiple  other nodes, providing multiple paths for data transmission. If one link fails, data can still  be routed through alternate paths without disrupting communication. In contrast, star  topology has moderate fault tolerance; while failure of individual nodes does not affect the  network, failure of the central hub or switch can cause the entire network to stop  functioning. Therefore, mesh topology is more robust and reliable in critical environments  compared to star topology.

| Feature | Mesh Topology | Star Topology |
| --- | --- | --- |
| Fault Tolerance | Very high because each node has multiple connections and alternative paths. | Moderate because all devices depend on the central switch or hub. |
| Link Failure | Failure of one link does not usually affect communication because another path is available. | Failure of a connection to one device affects only that device. |
| Central Device Failure | No central device is required, so there is no single point of failure. | Failure of the central hub/switch can stop the entire network. |
| Reliability | Highly reliable and suitable for critical networks. | Reliable for normal networks but less robust than mesh. |
| Cost | Expensive due to the large number of connections and cables required. | Less expensive and easier to implement. |
| Maintenance | More complex because of the large number of connections. | Easier to manage and troubleshoot. |

3.Bus topology is simple and easy to install because it uses a single backbone cable with  minimal connections, making it suitable for small networks. On the other hand, mesh  topology is highly complex to install as each node must be connected to every other node,  requiring a large number of cables and ports. This complexity increases significantly as the  number of devices grows, making mesh topology time-consuming and costly to implement  compared to the straightforward setup of bus topology.

| Feature | Bus Topology | Mesh Topology |
| --- | --- | --- |
| Installation | Simple and easy to install. | Complex and time-consuming to install. |
| Connections | Uses a single backbone cable with minimal connections. | Each node is connected to multiple or all other nodes. |
| Cabling | Requires less cable, making it economical. | Requires a large number of cables and ports. |
| Complexity | Low complexity. | High complexity. |
| Scalability | Easy to set up for small networks, but performance decreases as devices increase. | Becomes increasingly complex as more devices are added. |
| Cost | Low-cost and economical. | Expensive due to extensive cabling and hardware requirements. |
| Suitability | Suitable for small and simple networks. | Suitable for critical networks requiring high reliability. |

4.Hybrid topology is highly scalable because it combines two or more different topologies  (such as star, mesh, or bus), allowing the network to expand flexibly according to  organizational needs. New segments can be added without affecting the existing network  structure. In comparison, star topology is also scalable but to a limited extent, as it depends  on the capacity of the central device (switch or hub). Once the central device reaches its  port limit, expansion requires additional hardware. Thus, hybrid topology provides greater  scalability than star topology.

| Feature | Hybrid Topology | Star Topology |
| --- | --- | --- |
| Scalability | Highly scalable because different topologies can be combined and new network segments can be added easily. | Scalable to a certain extent, depending on the capacity of the central switch or hub. |
| Network Expansion | New segments can be added without significantly affecting the existing network. | Expansion may require an additional switch or hub when the existing device reaches its port limit. |
| Flexibility | Very flexible because it can combine star, mesh, bus, or other topologies. | Less flexible because all devices depend on the central device. |
| Cost | Generally more expensive due to the use of different networking devices and structures. | Usually less expensive and simpler to implement. |
| Complexity | More complex to design, configure, and maintain. | Easier to design and manage. |
| Suitability | Suitable for large organizations and networks with changing requirements. | Suitable for small and medium-sized networks. |

5.In terms of maintenance, star topology is relatively easy to manage because issues can be  quickly identified and isolated to a specific node or cable. Mesh topology, while highly  reliable, is difficult to maintain due to its complex structure and large number of  connections, making troubleshooting time-consuming. Bus topology is harder to maintain  than star because faults in the backbone cable can disrupt the entire network and are difficult  to locate. Hybrid topology has moderate maintenance complexity; while it benefits from  flexibility, managing multiple combined topologies requires careful monitoring and skilled  administration.

| Feature | Star Topology | Mesh Topology | Bus Topology | Hybrid Topology |
| --- | --- | --- | --- | --- |
| Maintenance | Easy to maintain and manage. | Difficult to maintain due to its complex structure. | Relatively difficult to maintain. | Moderately difficult to maintain. |
| Fault Identification | Easy to identify and isolate a faulty node or cable. | Troubleshooting is time-consuming because of many connections. | Faults in the backbone cable can be difficult to locate. | Requires careful monitoring of different network segments. |
| Effect of Failure | Failure of one cable usually affects only one node. | Alternative paths can keep communication active if a link fails. | Backbone failure can disrupt the entire network. | Depends on the topology used in each segment. |
| Administration | Simple and requires less technical expertise. | Requires skilled administration. | Simple structure but fault detection can be difficult. | Requires skilled administrators to manage multiple topologies. |
| Maintenance Cost | Relatively low. | High due to numerous connections. | Low to moderate. | Moderate to high depending on network complexity. |

RUBRICS FOR CALCULATION

| Criteria |  | Weightage Exemplary (4) Proficient (3) Developing (2) Beginning (1) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 15% | Demonstrates complete and in depth understanding of all concepts being compared | Good understanding with minor gaps | Basic understanding; some concepts unclear | Poor understanding of concepts |
| Comparison Depth | 20% | Provides detailed, insightful, and meaningful comparisons across multiple aspects | Adequate comparison with relevant points | Limited comparison; lacks depth | Minimal or incorrect comparison |
| Criteria Selection | 10% | Selects highly relevant and appropriate comparison parameters | Mostly relevant criteria | Some irrelevant or missing criteria | Poor or no criteria selection |
| Analytical Skills | 15% | Strong analysis with logical reasoning and clear justification | Good analysis with some justification | Basic analysis with weak reasoning | No clear analysis |
| Organization & Structure | 10% | Well-organized, clear, and logically structured | Generally organized | Somewhat disorganized | Poorly structured |
| Use of Examples / Evidence | 10% | Uses strong, relevant examples or data to support comparison | Uses some examples | Limited examples | No supporting examples |
| Clarity of Presentation | 10% | Very clear, concise, and easy to understand | Mostly clear | Somewhat unclear | Difficult to understand |
| Conclusion & Insights | 10% | Insightful conclusion with strong justification | Clear conclusion | Basic conclusion | No or weak conclusion |